AUTUMN END SEMESTER EXAMINATIONS-2023

SCHEME OF EVALUATION

SUBJECT: COMPUTER ORGANIZATION AND ARCHITECTURE(IT-21002)

Branch: IT, CSSE                              Total Marks: 50

Q1

[1 mark for correct answer. Partial marks can be awarded.]

[1 mark for correct answer with proper explaination]

(i)ADD R1,(R2)+    [3 memory references if 2nd operand is the destination]

(d)What is the use of WMFC signal in memory fetch operation?

The WMFC (Wait for Memory-Function-Completed) signal  causes the processor waits for the arrival of the MFC signal.When memory READ operation is completed then WMFC signal is generated.

[1 mark for correct answer. Partial marks can be awarded.]

(e )A processor has 180 distinct instructions and 40 general-purpose registers. A 32 bit instruction word has an opcode, two register operands and an immediate operand. How many bits are available for the immediate operand field?

| Opccode=8 | Register=6 | Register=6 | Immediate operand=12 |
| --- | --- | --- | --- |

[1 mark for correct answer with proper explaination]

(f)A processor advertised as having a 900 MHz clock is always provides better performance than a 700-MHz processor (True/False). Justify your answer.

False,because both processor may have a different value of S.

T=(N*S)/R

(g)How many 64 X 8 RAM chips are needed to provide a memory capacity of 2048 bytes?

No of chip=(2048*8) /(64*8)=32

[1 mark for correct answer. Partial marks can be awarded.]

(h)A cache has 95% hit ratio, an access time of 100ns on cache hit and access time of 800ns on cache miss. Compute the effective access time of cache memory.

Tavg=hc+(1-h)M

=o.95*100+0.05*800

=135ns

[1 mark for correct answer with proper explaination]

- Find the number of gate delays for obtaining C20(Carry-20) in the following 20-bit carry-look ahead adder built from five 4-bit adders.

- C20=3+2+2+2+2=11gate delay

[1 mark for correct answer. Partial marks can be awarded.]

- (j)Distinguish between cycle stealing and burst mode data transfer in DMA .

- (1)Burst mode DMA transfers data in a continuous sequence, while cycle stealing DMA temporarily takes control of the system bus to transfer smaller amounts of data before returning control to the CPU.

- (2) Burst mode is used for faster device,while cycle stealing  is used for slower device.

[1 mark for correct answer. Partial marks can be awarded.]

- SECTION-B

- Q2(a).Differentiate big endian and little endian address assignment schemes. Represent the number “-30” in little and big endian style. Assuming the memory locations starts at the address 3000 and the memory is byte-addressable memory organized in 32-bit words.

(b)Discuss the factors that affect the performance of the computer. If a 8GHz computer takes 7 clock cycles for ALU instructions, 11 clock cycles for branch instructions and 6 clock cycles for data transfer instructions. Then Find the total time taken by the computer to execute the program that consists of 10 ALU instructions, 5 branch instructions and 5 data transfer instructions

Q3(a)Write a program to evaluate the arithmetic statement:

X = (A-B+C*(D*E-F)) / (G+H*K)

i) Using a stack organized computer with zero-address operation instructions.

ii) Using an accumulator type computer with one address instructions.

iii) Using a general register computer with two address instructions

(b)What is the use of addressing mode? Difference between indirect and register indirect addressing mode. Write the assembly language code to execute the C-language statement A = *B, using indirect and register indirect addressing mode where B is a pointer variable.

Addressing modes are different ways to specify memory location from where data needs to be fetched for that instruction.

Indirect - In indirect addressing mode, memory location (address) specified in the instruction stores the address of memory location from where data needs to be fetched.

Register Indirect - In register indirect addressing mode, register specified in the instruction stores the address of memory location from where data needs to be fetched.

A = *B

Indirect addressing mode

MOVE (B),A      [2nd operand is the destination]

Register Indirect addressing mode

MOVE B,R1       [2nd operand is the destination]

MOVE (R1),A     [2nd operand is the destination]

[1 mark for addressing mode definition / use

1 mark for difference memory indirect / indirect and register indirect addressing modes

1 mark for assembly code of indirect mode

1 mark for assembly code of register indirect mode]

SECTION-C

Q4(a)

- PC=124   SP=100   TOS=[100]                   [1]

- PC=604   SP=96      TOS=124                     [1]

- PC=304   SP=92      TOS=616                     [1]

- PC=616    SP=96      TOS=124                    [1]

(b)Explain the 3-bus architecture inside CPU with neat diagram. Write the control signals for the following instruction. [10 Marks]

MUL  (R1) , R5

- MUL 5(R1),R5

- PCout,R=B,MARin,Read,Inc.PC

- WMFC

- MDAoutB,R=B,IRin

- Offset field of IRout,R1outB,Select A,ADD,MARin,Read

- WMFC

- R1outA,MDRoutB,Select A,MUL,R5in,end

- [Diagram and explanation  2mark and control step 2mark]

Q5(a)Write the sequence of control steps for the following instructions for single bus CPU organization

I. I1: ADD 10(R3), R4

II. I2: Branch<0 L1

III. I3: MUL -(R5), R5

Design the logic function for WMFC control signal with reference to the above instructions i.e. I1 to I3.

I1 :ADD 10(R3), R4

- PCout,MARin,READ,select4,ADD,Zin

- Zout,Pcin,Yin,WMFC

- Zout,IRin

- Offset field of IRout,Yin

- R3out,Seiect Y,ADD,Zin

- Zout,MARin,Read,

- R4out,Yin,WMFC

- MDRout,Select Y,ADD,Zin

- Zout,R4in.end

I2:Branch<0 L1

1.PCout,MARin,READ,select4,ADD,Zin

- 2.Zout,Pcin,Yin,WMFC

- 3.Zout,IRin

- Execution stage do not have memory operation

I3: MUL -(R5), R5

1.PCout,MARin,READ,select4,ADD,Zin

- 2.Zout,Pcin,Yin,WMFC

- 3.Zout,IRin

- 4.R5out,Select 4,SUB,Zin

- 5.Zout,R5in,MARin,Read

- 6.R5out,Yin,WMFC

- 7.MDRout,SelectY,MUL,Zin,R5in,end

WMFC=T2+I1.T7+I3.T6+………..

[Control step for each instruction 1mark and design of logic circuit  1mark]

(b)Write the function of control unit. Explain the following terms related to micro-programmed control unit design:

(i) Micro program counter (ii) Micro Routine

- (iii) Micro Instruction (iv) Control Store

[1 mark for each correct answer. Partial marks can be awarded.]

Q6(a)Write the steps for multiplication according to Booth Multiplication.Explain with the example to multiply 9 with -4.

- [Both the method be considered.Partial marks can be awarded]

- (b)

- Classification of data transfer.            [1]

- Explanation of DMA.                         [3]

- [Partial marks can be awarded]

- 7(a)

- Differentiation of SRAM and DRAM.         [2]

- Design                                                           [2]

- [Partial marks can be awarded]

- No of chip=64                                    Decoder size=4:16

- Common address line=A0 to A18     Chip selection line=A19 to A22

- Each word line have 4 no of chip

- Data line partition:

- D0-D7   D8-D15  D16-D23   D24-D31

- (b)

- Importance of cache memory in memory organization.                 [2]

- (i)Direct mapping

| - TAG=4 | - BLOCK=7 | - WORD=5 |
| --- | --- | --- |

- (ii)4-way set-associative mapping

| - TAG=6 | - SET=5 | - WORD=5 |
| --- | --- | --- |

Q8(a)Compare direct, set associative and fully associative mapping techniques.

Consider a 2-way set associative cache memory with 4 sets and a total 8 cache blocks (0-7) and a main memory with 128 blocks (0-127). What memory block will be present in the cache after the following sequence of memory references if LRU and FIFO policy is used for block replacement?

0  5  3  9  7  0  16  55

Comparision                                                                                                       [2]

Solution of problem                                                                                            [2]

[Partial marks can be awarded]

- Direct Mapped: Each block has only one place that it can appear in the cache.

Cache Block no = (Memory Block number) mod (No. of blocks in cache)

- Fully associative: Each block can be placed anywhere in the cache.

- Set associative:  Each block can be placed in a restricted set of places in the cache.

  - If there are n blocks in a set, the cache placement is called n-way set associative ( eg. 2 block in a set means 2 way set associative)

Cache Set no. = (Memory Block address) mod (No. of sets in cache)

(b)

- Dividend=13        Divisor=4

- (13)10=(1101)2        (4)10=(0100)2

- Initial: A=0000   Q=1101(Dividend)

- M=0100(Divisor)     M’=1100

- SC=4(total no of cycle)

- RESULT: Quotient=0011 (3)

- Remainder=0001 (1)

- In Non-Restoring 2nd step is not required.

- Restoring                                              [2]

- Non-Restoring                                      [2]

- [Partial marks can be awarded]